SECTION 09830

LOW FREQUENCY SOUND ABSORBER (MODEX™ PLATE 2.0)

PART 1 GENERAL

  - SECTION INCLUDES

A.	Low Frequency Sound Absorber (Model Modex™ Plate 2.0)

  - RELATED SECTIONS

A.	Section 06400 – Architectural Woodwork

  - DEFINITIONS

A.	The Modex™ Plate 2.0 shall offer absorption from 50 to 300 Hz, utilizing a damped plate resonator with added porous absorption.  The assembly shall be within a metal constructed cabinet and finished in low sheen black.

  - SYSTEM DESCRIPTION

- Design Requirements: Surface or corner applied porous materials lose efficiency at low frequencies, because the particle velocity or air movement associated with these long wavelengths is low.  The Modex™ Plate is pressure activated and is most effective in high pressure dihedral or trihedral surface locations. The system shall offer low and mid frequency absorption via a mass-spring mechanism.

- Performance Requirements:

- Random Incidence Sound Absorption Coefficients (a): Tested by independent, accredited, NVLAP facility according to ISO 354 for a type A mounting of the unit:

| f (Hz) | Absorption |
| --- | --- |
| 50 | 0.73 |
| 63 | 0.67 |
| 80 | 0.73 |
| 100 | 0.75 |
| 125 | 0.86 |
| 200 | 0.79 |
| 250 | 0.59 |
| 315 | 0.56 |
| 400 | 0.39 |
| 800 | 0.29 |
| 1250 | 0.26 |
| 2000 | 0.22 |

  - SUBMITTALS

- Product Data: Submit standard manufacturer product cutsheet showing product, standard dimensions and selected options.

- Design Data / Test Reports: Submit sound absorption coefficients.

  - QUALITY ASSURANCE

- Flame Spread / Smoke Developed Characteristics: Components tested by independent, accredited NVLAP facility according to DIN 4102 B1. Class A rating.

  - DELIVERY STORAGE AND HANDLING

- Packing, Shipping, Handling and Unloading: Modex Plates are susceptible to damage and shall be removed from packaging and handled with care.  Units shall be carried by a minimum of 2 people.  Units shall never be set down on finished fabric faces, only on backside.

- Storage and Protection: Store Modex Plates in original packaging until ready to install.  Store Modex Plates in temperature and humidity controlled conditions for 24 hours prior to installation and protect from moisture and infestation.

- Acceptance at Site: Installation constitutes complete acceptance.

  - PROJECT CONDITIONS

- Project Environmental Requirements: Installation area shall be enclosed, protected from weather and be temperature and humidity controlled prior to unpacking and installing Modex Plates.

PART 2 PRODUCTS

  - DISTRIBUTOR

A.	RPG Acoustical Systems, LLC, 99 South St, Passaic, NJ 07055 (telephone) 973-916-1166, http://www.rpgacoustic.com.

  - MATERIALS

- Unit Dimensions: Width 32-1/2” x Height 48-1/2” x 4-1/4” Depth

- Weight: 50 lbs (approximately 5 lbs per square foot)

- Finish: Standard finish is a low sheen black. Units can also be custom painted, or fabric wrapped.

  - ACCESSORIES

A.	Wall Cleats: Units shall be supplied with (2) metal mounting cleats running the inside width of the perimeter frame.

PART 3 EXECUTION

  - INSTALLERS

A.	Only qualified installers with 3 years (minimum) experience installing similar products shall install diffusors.

  - EXAMINATION

A.	Site Verification of Conditions: Commencement of installation constitutes acceptance of prior work.

  - PREPARATION

- Protection: Protect surrounding work so as to avoid damage during installation of diffusors.

- Surface Preparation: Inspect substrate and ensure surface is flat, plumb, clean and dry without protruding elements that would otherwise interfere with installation.

  - INSTALLATION

- Install wood panels as shown and detailed in the architectural drawings and according to manufacturer’s guidelines and industry standards.

- Install metal wall cleats provided with Modex Plate.

- Install Modex Plate by placing unit over cleats and slide down until firmly in place..

  - CLEANING AND PROTECTION

A.	After installation, protect panels against dirt, water and contact that would puncture, snag, tear or otherwise damage panel fabric.

END OF SECTION